Tool to correct the river network

Version 0.9  PB 4/3/2024

Version 0.91  PB 28/11/2024

Folder: Tool_to_correct_...

But might be faster if you copy the whole folder to your local drive.

The cor_Danube_1min1.mxd has stored relative pathnames – so it should find the correct shape and raster files (but you have to change the pathnames in the model builder)

- Main files and folders

cor_Danube_1min1.mxd: 	ArcGIS file

ldd_points_danube.shp:	Shape with arrows for directions of the river network

Correct_LDD.tbx:		Toolbox for ArcGIS to create shape and rasters from point shape

LDD_corrected2: Folder with results (shape and rasters)

Shapes: Folder with shape files e.g. EU_hydro river network

Raster: Raster .tif files e.g. ldd.tif (river network), upstream area, cellarea

- Work with the tool

2.1 Create point shape file from existing rivernetwork raster (ldd.tif)

- load ../TOOL_to_correct_Danube/Correct_LDD.tbx into the toolbox

  - open ArcToolbox, right click on ArcToolbox -> Add toolbox

- run 1_create_ldd_point

  - ArcGis is not overwriting files by default. If you want to overwrite your files you have to run: Correct_LDD/Script first

  - You might have to change the path. ArcGis has a problem with relative paths.
You have to change the last part of the path (or overwrite files)

- This will create a point shape of all grid points:

e.g. LDD_correct3/ ldd_points_morava.shp

- if you created a ldd_points_...

  - move ldd_points_.. to HighReszoomIn

  - import Symbology

You now have a point shape file with direction indicators of each cell

2.2 Changing the river direction in the shape ldd_points_danube

- right click ldd_points_danube -> selection -> Make this the only selectable layer

- customize -> toolbars -> editor

- Editor -> start editing _> ldd_points_danube

- select single point -> open attribute table

- change grid_code (eg. from 1 (SW) to 2 (South)
	The direction is the same as the extended keyboard
7 8 9
4    6
1 2 3
and the 5 is indicating a sink (last point of a basin)
After selecting the next - check if the arrow point in the right direction

- select next point (arrow) and change. Repeat and repeat and repeat

- Editor -> save edits (will take some time)

Result:

A corrected point shape file with direction indicators

Remarks:

Best check your river network with other data:

(here on the right: OpenStreetMap and EU_Hydro river network)

Even better: Country data (Gewaessernetz Austria)

2.3 Creating a new raster river network (LDD – local drainage direction)

- run 2_correct_LDD_tiff_shape

  - ArcGis is not overwriting files by default. If you want to overwrite your files you have to run: Correct_LDD/Script first

  - You might have to change the path. ArcGis has a problem with relative paths.
You have to change the last part of the path (or overwrite files)

  - In case you want to permanently change the path names you have to edit the model build (right click on 2_correct_LDD_tiff_shape and Edit)

Results:

- ldd_repair.tif    correct LDD -> used for CWatM

- ldd_repaired_ArcGIS.tif   LDD with ArcGis convention of neighbor points

- ldd_deg.tif: LDD with degree as conventionneighbor points 
e.g. ldd _repair value 7: is pointing to northeast -> ArcGIS: 32 -> degree: 315

- watershed_morava.tif -> watershed of the basin: all points included in the watershed (here you can see, if you excluded some gridcell, by an accidently circular round)

- ldd_lines.shp : connection lines of grid points

- FlowAcc.tif: upstream area calculated with ArcGis

2.4 Creating a line shape which shows the direction of the river network

- run 3_create_river_layer

  - change the pathnames

  - the shape file: ldd_lines.shp has to be loaded

- if you created a new river_from_LDD2

  - move river_from_LDD2 to HighReszoomIn

  - import Symbology

- remarks:

- try to avoid number <0 and number >9. This will crash the network.

If you are unsure, that you put a e.g. 23 accidently in:
sort ldd_points_danube biggest first then lowest first to see if you put in a number >9 or <1

- try to avoid circular round, e.g. a grid cell is pointing to another gridcell pointing back to the original gridcell (can be bigger loops, too)